COURSE CODE: ECA1503

COURSE NAME: TRANSMISSION LINES AND WAVE GUIDES

NAME: G. Bhanu Siva Mani Ram

Reg.No:192412471

| 2 | Design a rectangular waveguide operating in the X-band using CST. Simulate the TE(_{10}) mode and evaluate the cutoff frequency. |
| --- | --- |

AIM

To design and simulate a WR-90 rectangular waveguide operating in the TE₁₀ mode using CST Studio Suite and determine its cutoff frequency and S-parameter performance.

2. PROCEDURE

- Open CST Studio Suite and create a new microwave project.

- Select Time Domain Solver and set the frequency range to 6–12 GHz.

- Create a rectangular waveguide with:

  - Width a=22.86mm

  - Height b=10.16mm

  - Length = 100 mm

- Assign copper as the conducting material.

- Add Waveguide Port 1 at the input and Port 2 at the output.

- Select the TE₁₀ mode for excitation.

- Set suitable open/metal boundary conditions and generate the mesh.

- Run the CST simulation.

- Observe the electric-field distribution and S11/S21 parameters.

OUTPUT:

3. RESULT

The theoretical cutoff frequency for the dominant TE₁₀ mode is:

fc=c2a
fc=3×108222.86×10-3
fc=6.56 GHz

The simulation confirms TE₁₀ mode propagation above 6.56 GHz. The electric-field distribution shows the expected TE₁₀ field pattern, while the S-parameter plots can be used to evaluate reflection and transmission.